Tempmate M1 Temperature Logger Instructions

General Information

The Tempmate M1 is a reusable temperature logger with display and integrated USB connection. This logger has been pre-programmed by us and is ready to use immediately.

Starting the Logger

- Press and hold the left button (scroll) for 5 seconds

- The display will show "REC"

- The logger will now begin recording temperature data

Viewing Status During Use

A short press on the left button will cycle through the following information on the display:

- Current status of the logger

- Minimum recorded temperature

- Maximum recorded temperature

- Number of recorded measurements (LOG)

- Current temperature

Reprogramming the Logger

The Tempmate M1 can be fully reconfigured using the free Tempbase-2 software. With it, you can:

- Set start and stop conditions

- Adjust measurement intervals

- Define alarm thresholds

- Assign a name or ID to the logger

- Modify report and display settings

The full manual is available via the Help function in the software.

Reading Measurement Data

After use, the logger can be read in two ways:

- Via the USB port on your computer

  - The logger automatically generates a PDF report and additional files such as .TXT and .CSV

- Via the Tempbase-2 software

  - For advanced analysis and data export

  - Includes a visual configuration screen for the logger

Download the software here:
Tempbase 2 – Software link (ZIP)

Happy measuring!
Dataloggers.shop
www.dataloggers.shop
✉ mail@tempro.be